# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Supply of greenhouse structure and related equipment.

Ref. number: HR-BA-ME00486 07-347/26/PP6

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 1. | Greenhouse Structure – VENLO Type - Structure type: Venlo, width 8.00 m (2 spans × 4.00 m) - Length: 5 bays × 4.00 m, total 20.0 m - Gutter height: 5.00 m (from foundation level) - Columns and cross‑beams: hot‑dip galvanized steel - Roof structure: aluminium - Gutters: aluminium, with vertical PVC drainage pipes Ø110 mm - Vertical glass supports: aluminium - Roof vents: both sides, 4 complete sets, operated via motor‑reducer - Side sliding door: 2.00 × 2.50 m (W × H) - Roof cladding: single‑layer tempered glass, 4 mm - Side and end wall cladding: single‑layer tempered glass, 4 mm |  |  |  |
| 2. | Internal Shading System – Horizontal Screen - Horizontal energy-saving and shading screen - Minimum 47% energy saving and minimum 55% shading - Cable-driven system with motor-reducer - Field width: 9.00 m - Field length: 4.30 m - Number of fields: 5 - Controlled via climate computer |  |  |  |
| 3. | Climate Computer with Weather Station - Process‑based climate computer with basic control functions: - ventilation, heating, CO₂ dosing, circulation fans, shading - Weather station including sensors for: - wind speed, wind direction, temperature, rainfall, solar radiation - Indoor sensors: temperature and relative humidity - Weather station mounted on stainless‑steel bracke |  |  |  |
| 4. | Submergible Benches - Dimensions: 1500–1600 × 3900–4100 mm - Frame: aluminium profile, height 124 mm - Corner joints: aluminium, assembled with pop rivets - PVC finishing elements - Trays included, with special adhesive and filter - Each bench equipped with inlet for flooding and drainage - One quick‑valve per bench - Uniform filling system - Maximum load capacity: 60–80 kg/m² |  |  |  |
| 5. | Fertigation Unit - Stainless‑steel structural frame with adjustable legs - Mixing tank 150–180 L with filling valve - Three dosing channels, 600 L/h (2 × nutrient solution A/B + 1 × pH), each with flow meter - Electrical cabinet with control relays and interface - Measuring lines with pH and EC sensors - Main pump: up to 5 m³/h, up to 3.5 bar - Base pumps - All PVC materials, manometers, valves, pressure regulators, cables and required fittings - Tanks for A and B solutions (up to 250 L each) - Acid tank (up to 250 L) - Irrigation controlled automatically via climate computer. - System supports ebb‑and‑flow benches and hanging gutters. |  |  |  |
| 6. | Hanging Gutter System (16 m) - Tomato growing gutter - Starting caps - End caps with outlet - Plant support hanging system |  |  |  |
| 7. | Irrigation and Drainage System - Main pipeline: PVC DN 50 - Drainage pipeline: PVC - Drip irrigation line: PVC Ø20 mm, white - Drippers spaced every 20 cm - Dripper flow rate: 3–4 L/h |  |  |  |
| 8. | Installation of Greenhouse and Equipment - Includes all machinery, tools, labour and materials required for complete installation |  |  |  |
| 9. | Transport - Transport of all equipment to the contracting authority’s site |  |  |  |

All supplies, works, installations and commissioning activities must be carried out in full accordance with the project and the approved project documentation. The Contractor shall ensure complete compliance with all project requirements and shall be fully responsible for the correct implementation of all project-related technical solutions. Any deviation from the project documentation is not permitted unless explicitly approved in writing by the Contracting Authory.